Periodic Inspection of Exterior Walls and Appurtenances

Façade Inspection Safety Program 
(FISP, Formerly Local Law 11/98)

Initial Cycle 9(A, B or C) FISP Report #TR6-919144-9A-I1

PROPERTY ADDRESS

Address

aka Address

City, NY ZIP

OWNER

Hotel Z LIC Nominee, LLC c/o Jones Lang LaSalle

11-04 43rd Avenue

Long Island City, NY 11101

LICENSED PROFESSIONAL

Name

License #

Company Name

Address

City, State Zip code

# Executive Summary

General Description:

The property located at 43-01 11th Street (aka 11-04 43rd Avenue), Long Island City, New York is a 10-story commercial building with a basement level, recreational rooftop lounge area, 2nd floor mechanical roof and mechanical bulkhead. The façade consists of exterior insulation and finish system (EIFS) panels at each floor level and continuous windows. The building is located at the southeast corner of 43rd Avenue and 11th Street in Queens.

Note: There are two buildings erected on the tax lot (Block: 444, Lot: 14); the other structure is classified as a parking garage with a separate Owner of Record.

Scope of Inspection:

The scope of our inspection included visual examinations of the (entire building at all four building elevations) using high-resolution binoculars and a digital camera with a zoom lens from the ground level. Close, physical inspections of the facade were performed from (industrial rope access along the west elevation (11th Street, window lines W1-W2, W5-W6, and W9-W10), south elevation (43rd Avenue, window line S1 plus one additional drop), and east elevation (window line E2)). Close, physical inspections were also conducted from the main roof and from the 2nd floor mechanical roof. Refer to the Plot Plan - Key Plan (uploaded to DOB NOW as a supporting document) for the locations noted within this report.

No probes were performed as part of the facade inspection, as the exterior walls are comprised of a curtain wall system (window walls) and EIFS.

Summary of Findings:

As a result of the visual and physical inspections of the exterior walls and appurtenances, we observed new Cycle 9A SWARMP (Safe with a Repair and Maintenance Program) façade conditions at the south, west, and east elevations, including cracked EIFS panels and failing floor tile edging. At the time of the critical examination, no signs of distress or excessive settlement were observed. All observed conditions were thoroughly sounded, where accessible. No movement was observed.

All building appurtenances were observed to be in SAFE and code-compliant condition at the time of inspection. Refer to Section GA for further detail.

The Owner indicated that there are no reports of water leaks from the exterior walls or roof surfaces. No obvious signs or evidence of active water infiltration were observed.

Conclusions and Recommendations:

In accordance with 1 RCNY §103-04, the building is classified as SWARMP (Safe with a Repair and Maintenance Program). Sullivan Engineering, A Rimkus Company, recommends that a comprehensive façade restoration effort be implemented to address the deficiencies listed in this report within two years, by October 31, 2024. Refer to Section K of this report for further detail.

# Building Information

Control Number:			919144

Address:				11-04 43rd Avenue

AKA:					43-01 – 43-07 11th Street;

11-02 – 11-16 43rd Avenue

Borough / Zip:				Queens, 11101

Cross Streets:				11th Street, 12th Street

BIN:					4436814

Additional BINs:			4436813

Block:					444

Lot:					14

Special District:			N/A

Landmarks Historic District:		N/A

Additional Designation(s):		IBZ – Industrial Business Zone

# Owner Information

Owner Type:				Private

Owner Name/Entity:			Hotel Z LIC Nominee, LLC (for BIN: 4436814)

Bernardo Crane Service, Inc. (for BIN: 4436813)

Owner Representative:		Michael Mankowich

Management Company:		Jones Lang LaSalle

Email:					michael.mankowich@am.jll.com

Address:				330 Madison Avenue, New York, NY  10017

Phone:					(646) 630-4900

# Building Description

Number of Stories:			10

Height:					110 feet

Number of Balconies:			0

Building Use:				Commercial

Certificate of Occupancy:		N/A

Lot Dimensions:			165 feet x 95 feet

Building Dimensions:			150 feet x 43 feet

Approximate Year Built (Age):	c. 2011 (11 years)

Exterior Wall Type:			Curtain Wall

Exterior Wall Material(s):	Glass, EIFS

# Detailed Description of any Settlements, Repairs, or Revisions to Exterior Enclosures Since Previous Report

The Owner advised that there have been no previous reports of any significant settlement of the building’s structure. At the time of the critical examination, no signs of distress or excessive settlement were observed. No movement was observed.

This is the first FISP report submission for the building. There have been no previous exterior repair campaigns or FISP filings for the property.

Per the New York City Department of Buildings (NYC DOB) Building Information Search (BIS) and DOB NOW websites, NYC DOB Job No. Q00363567 was filed in 2020 by Mottola Rini Engineers for the installation of HVAC equipment at the ground floor and roof level (Permit No. Q00363567-I1-MS, signed-off 11/18/2020).

# Procedures Used in Making Critical Examination

The scope of our inspection included visual examinations of the entire building at all four building elevations using high-resolution binoculars and a digital camera with a zoom lens from the ground level. There are existing 1- and 2-story buildings along the north and east elevations which prevented observation of a portion of the facade (42-36 43rd Avenue and 11-17 43rd Avenue, respectively).

Close, physical inspections of the facade were performed from industrial rope access along the west elevation (11th Street, window lines W1-W2, W5-W6, and W9-W10), south elevation (43rd Avenue, window line S1 plus one additional drop), and east elevation (window line E2). Close, physical inspections were also conducted from the main roof and from the 2nd floor mechanical roof. Refer to the Plot Plan - Key Plan (uploaded to DOB NOW as a supporting document) for the locations noted within this report.

No probes were performed as part of the facade inspection, as the exterior walls are comprised of a curtain wall system (window walls) and EIFS (Exterior Insulation and Finish System).

# Extent and Location of Physical Examinations

QEWI: 							License Person Name

Person(s) who conducted inspection: 		Inspector Name

Relationship to QEWI: 				Self

Details of Inspection(s): 	Wednesday, September 14, 2022 – Visual inspection, main roof, and close-up inspections via rope access

Wednesday, October 19, 2022 – Final visual inspection by QEWI

Rigger:            	Structural Preservation Systems, LLC;

35-21 Vernon Boulevard, Long Island City, NY 11106 (Rigger License No. 006257)

# Description, Location(s), and Classifications of Conditions Observed

At the time of inspection, no signs of movement or excessive settlement were observed. All accessible conditions were thoroughly sounded with a rubber mallet and hammer, and all loose EIFS material was removed at the time of inspection.

Watertightness of Exterior Surfaces:  The Owner indicated that there are no reports of water leaks from the exterior walls or roof surfaces. No obvious signs or evidence of active water infiltration were observed along the facade; however, visual observations of the interior building spaces were excluded from the scope of the inspection.

The following chart contains a comprehensive list of the deleterious façade conditions identified during the inspection. Refer to Appendix 2, Plot Plan - Key Plan for mapping of the locations (window lines) noted below.

| Item | Description | Location ID (Window Line) | Floor(s) | Repair Timeframe (Years) | Rec. Completion Date | Cause | Photo(s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| West Elevation (SWARMP Conditions) |  |  |  |  |  |  |  |
| W.1 | Deteriorated EIFS | W1 - W2 | 4, 5 | 3 | 10/31/25 | Age and weathering | 29-33 |
| W.2 | Deteriorated EIFS | W5, W6 | 5, 6, 7 | 3 | 10/31/25 | Age and weathering | 34-36 |
| South Elevation (SWARMP Conditions) |  |  |  |  |  |  |  |
| S.1 | Failing tile edging | S1 | 2 - 10 | 2 | 10/31/24 | Age and weathering | 37, 38 |
| S.2 | Cracked EIFS | S1 | 1 | 2 | 10/31/24 | Age and weathering | 39 |
| East Elevation (SWARMP Conditions) |  |  |  |  |  |  |  |
| E.1 | Failing tile edging | E1 - E2, E5 - E6 | 2 - 10 | 2 | 10/31/24 | Age and weathering | 40 |
| E.2 | Cracked EIFS | E1, E6 | 10 | 2 | 10/31/24 | Age and weathering | 41, 42 |
| E.3 | Vertical cracks in EIFS | E1 | 1 | 2 | 10/31/24 | Age and weathering | 43 |
| North Elevation |  |  |  |  |  |  |  |
| No UNSAFE or SWARMP conditions were observed. |  |  |  |  |  |  |  |

# GA. Appurtenances

A critical examination of all exterior appurtenances was conducted, including the following (condition of each appurtenance noted in parenthesis):

- Parapets (SAFE)

- Copings (SAFE)

- Glass parapet panels and railings – main roof (SAFE)

- Vents and louvers (SAFE)

- Architectural (exposed) beams - east elevation (SAFE)

- Pipe penetrations (SAFE)

- Conduit (SAFE)

- Window frames (SAFE)

- Tile sills and edging (SWARMP)

- Exterior light fixtures (SAFE)

- Security cameras (SAFE)

- Main entrance canopy and anchorage (SAFE)

- Bulkhead canopy/awning (SAFE)

- Planters (SAFE)

All exterior appurtenances were observed to be in SAFE and code-compliant condition, with the exception of the failing tile edging observed along the south and east elevations. The tile edging has been cited as a Cycle 9A SWARMP condition, as noted in Section G of this report.

There are no balconies, enclosures, or fire escapes associated with the building.

# GB. Balcony, Guardrail, Handrail, and Parapet Statement

There is a recreational lounge area located at the main roof, which is accessible to building occupants. The glass parapet panels along the main roof perimeter were measured to be 61.5 inches in height from the walking surface. Refer to Photo Appendix - No. 8 through 18.

There is a mechanical roof at the 2nd floor along the north, south, and east elevations. The parapets at the mechanical roof were measured and found to exceed the 42-inch height requirement of the 2014 New York City Building Code. Refer to Photo Appendix - No. 19 and 20.

There is also a lower mechanical roof at the 1st floor of the west elevation, adjacent to the main entrance awning. This roof has a low parapet; however, its surface is inaccessible. Refer to Photo Appendix - No. 21 and 22.

The bulkhead roofs are inaccessible. There are no parapets or railings along their perimeters.

There are no balconies, enclosures, or fire escapes associated with the building. The existing guards were observed to be compliant with the code under which they were constructed, the 2008 New York City Building Code.

- Causes of Conditions Observed

The observed façade conditions appear to be the result of typical age and weathering, including deterioration due to freeze/thaw cycles. Please refer to the conditions chart in Section G of this report for the cause(s) of each identified Cycle 9A FISP condition.

- Status Report of Exterior Maintenance

No exterior or façade-related work is underway at this time.

# JA. 	Comparison of Current Conditions with Conditions from Previous Filings

Prior Filing Cycle:			N/A

Prior Filing Status:			N/A

Current Filing Cycle:			9A

Current Filing Status:			SWARMP

Are any conditions repeated from the previous filing?		No

This is the property’s first Façade Inspection & Safety Program (FISP) report filing. There were no previously identified FISP conditions.

# JB. 	Work Permit Numbers, Status, and Sign-off Dates

Since the completion of the building’s construction, no facade-related work has been conducted, per the New York City Department of Buildings (NYC DOB) Building Information Search (BIS) and DOB NOW websites.

NYC DOB Job No. Q00363567 was filed by Mottola Rini Engineers for the installation of HVAC equipment at the ground floor and roof (Permit No. Q00363567-I1-MS, signed-off 11/18/2020).

# JC. 	Building and Property Violations

Per DOB BIS and DOB NOW, there are no open ECB violations.

- Recommendations for Repairs or Maintenance

Sullivan Engineering, a Rimkus Company, recommends that a comprehensive façade restoration effort be implemented to address the deficiencies listed in the report (refer to the conditions chart in §G of this report for a complete list of the identified conditions and the copy uploaded as “Other” supplemental documentation).

The recommended repairs shall include removal and replacement of the tile edging and repairs to the existing EIFS panels. We recommend that the Cycle 9A SWARMP repairs be completed within two years, by October 31, 2024.

- Required Work Permits

In accordance with 1 RCNY §101-14, a work permit will be required to perform the recommended EIFS and tile edging repairs. A sidewalk shed permit will also be required to install protection along the public right-of way along the west elevation (11th Street).

- Current Report Classification

# Per 1 RCNY §103-04, the building is classified as SWARMP (Safe with a Repair and Maintenance Program).

- Seal & Signature

This report is intended solely to document conditions observed during the critical façade inspection. It is not to be used in lieu of construction documents or the proper specifications and drawings required for alterations, renovations, or repairs.

QEWI:	 	Name

License #

Date:		Date

- Appendices

- Plot Plan (Key Plan and Location Diagram)

- Elevation Photographs #1 through #10, #19 through #22

- Detailed Photographs #11 through #43

- QEWI Photographs #1, #27, and #28

- Seal and Signature (DPL-1)

- Other – Cycle 9A FISP Conditions Chart